Code used in Arduino:
// ====== ROS2 ARDUINO BRIDGE (FINAL VERSION) ======

// Receives linear & angular velocities from Raspberry Pi via serial

// and drives two motor drivers (Left + Right motor groups)

#define ENA_LEFT 5

#define IN1_LEFT 6

#define IN2_LEFT 7

#define ENB_LEFT 9

#define IN3_LEFT 10

#define IN4_LEFT 11

#define ENA_RIGHT 3

#define IN1_RIGHT 4

#define IN2_RIGHT 8

#define ENB_RIGHT 12

#define IN3_RIGHT 13

#define IN4_RIGHT 2

// Robot parameters

float wheel_base = 0.20;     // Distance between left and right wheels (meters)

float max_speed = 0.3;       // Maximum linear speed (m/s) - adjust per robot

int pwm_deadband = 60;       // Minimum PWM to overcome friction

float speed_gain = 2.0;      // Scale-up factor for responsiveness

void setup() {

Serial.begin(9600);

// Left motor driver pins

pinMode(ENA_LEFT, OUTPUT);

pinMode(IN1_LEFT, OUTPUT);

pinMode(IN2_LEFT, OUTPUT);

pinMode(ENB_LEFT, OUTPUT);

pinMode(IN3_LEFT, OUTPUT);

pinMode(IN4_LEFT, OUTPUT);

// Right motor driver pins

pinMode(ENA_RIGHT, OUTPUT);

pinMode(IN1_RIGHT, OUTPUT);

pinMode(IN2_RIGHT, OUTPUT);

pinMode(ENB_RIGHT, OUTPUT);

pinMode(IN3_RIGHT, OUTPUT);

pinMode(IN4_RIGHT, OUTPUT);

stopMotors();

Serial.println("✅ Arduino ready to receive commands...");

}

void loop() {

if (Serial.available()) {

String input = Serial.readStringUntil('\n');

input.trim();

if (input.length() > 0) {

float linear = 0.0, angular = 0.0;

int commaIndex = input.indexOf(',');

if (commaIndex > 0) {

linear = input.substring(0, commaIndex).toFloat();

angular = input.substring(commaIndex + 1).toFloat();

// Differential drive kinematics

float left_speed = linear - (angular * wheel_base / 2);

float right_speed = linear + (angular * wheel_base / 2);

// Scale up to improve response

left_speed *= speed_gain;

right_speed *= speed_gain;

// Convert to PWM (0–255)

int left_pwm = constrain(mapFloat(abs(left_speed), 0, max_speed, 0, 255), 0, 255);

int right_pwm = constrain(mapFloat(abs(right_speed), 0, max_speed, 0, 255), 0, 255);

// Apply deadband threshold

if (left_pwm < pwm_deadband) left_pwm = 0;

if (right_pwm < pwm_deadband) right_pwm = 0;

// Debug info (optional)

Serial.print("Linear: "); Serial.print(linear, 3);

Serial.print("  Angular: "); Serial.print(angular, 3);

Serial.print("  L_PWM: "); Serial.print(left_pwm);

Serial.print("  R_PWM: "); Serial.println(right_pwm);

// Apply direction control

if (left_speed > 0) forwardLeft(left_pwm);

else if (left_speed < 0) backwardLeft(left_pwm);

else stopLeft();

if (right_speed > 0) forwardRight(right_pwm);

else if (right_speed < 0) backwardRight(right_pwm);

else stopRight();

}

}

}

}

// ====== Motor Control Helper Functions ======

void forwardLeft(int pwm) {

digitalWrite(IN1_LEFT, HIGH);

digitalWrite(IN2_LEFT, LOW);

analogWrite(ENA_LEFT, pwm);

digitalWrite(IN3_LEFT, HIGH);

digitalWrite(IN4_LEFT, LOW);

analogWrite(ENB_LEFT, pwm);

}

void backwardLeft(int pwm) {

digitalWrite(IN1_LEFT, LOW);

digitalWrite(IN2_LEFT, HIGH);

analogWrite(ENA_LEFT, pwm);

digitalWrite(IN3_LEFT, LOW);

digitalWrite(IN4_LEFT, HIGH);

analogWrite(ENB_LEFT, pwm);

}

void stopLeft() {

digitalWrite(IN1_LEFT, LOW);

digitalWrite(IN2_LEFT, LOW);

digitalWrite(IN3_LEFT, LOW);

digitalWrite(IN4_LEFT, LOW);

analogWrite(ENA_LEFT, 0);

analogWrite(ENB_LEFT, 0);

}

void forwardRight(int pwm) {

digitalWrite(IN1_RIGHT, HIGH);

digitalWrite(IN2_RIGHT, LOW);

analogWrite(ENA_RIGHT, pwm);

digitalWrite(IN3_RIGHT, HIGH);

digitalWrite(IN4_RIGHT, LOW);

analogWrite(ENB_RIGHT, pwm);

}

void backwardRight(int pwm) {

digitalWrite(IN1_RIGHT, LOW);

digitalWrite(IN2_RIGHT, HIGH);

analogWrite(ENA_RIGHT, pwm);

digitalWrite(IN3_RIGHT, LOW);

digitalWrite(IN4_RIGHT, HIGH);

analogWrite(ENB_RIGHT, pwm);

}

void stopRight() {

digitalWrite(IN1_RIGHT, LOW);

digitalWrite(IN2_RIGHT, LOW);

digitalWrite(IN3_RIGHT, LOW);

digitalWrite(IN4_RIGHT, LOW);

analogWrite(ENA_RIGHT, 0);

analogWrite(ENB_RIGHT, 0);

}

void stopMotors() {

stopLeft();

stopRight();

}

float mapFloat(float x, float in_min, float in_max, float out_min, float out_max) {

return (x - in_min) * (out_max - out_min) / (in_max - in_min) + out_min;

}

Code used in fake_odom.py:

import rclpy

from rclpy.node import Node

from nav_msgs.msg import Odometry

from geometry_msgs.msg import Pose, Twist

import time

class FakeOdom(Node):

def __init__(self):

super().__init__('fake_odom_publisher')

self.publisher = self.create_publisher(Odometry, '/odom', 10)

self.timer = self.create_timer(0.1, self.publish_odom)

self.x = 0.0

self.y = 0.0

self.th = 0.0

def publish_odom(self):

msg = Odometry()

msg.header.stamp = self.get_clock().now().to_msg()

msg.header.frame_id = 'odom'

msg.child_frame_id = 'base_link'

# simulate forward motion

self.x += 0.01

msg.pose.pose.position.x = self.x

msg.pose.pose.position.y = self.y

msg.pose.pose.orientation.w = 1.0

msg.twist.twist.linear.x = 0.1

msg.twist.twist.angular.z = 0.0

self.publisher.publish(msg)

self.get_logger().info(f'Publishing fake odom: x={self.x:.2f}')

def main(args=None):

rclpy.init(args=args)

node = FakeOdom()

rclpy.spin(node)

node.destroy_node()

rclpy.shutdown()

if __name__ == '__main__':

main()

Code used in Arduino_bridge_node.py

import rclpy

from rclpy.node import Node

from geometry_msgs.msg import TwistStamped

import serial

import time

class ArduinoBridge(Node):

def __init__(self):

super().__init__('arduino_bridge')

# Subscribe only to TwistStamped

self.subscription = self.create_subscription(

TwistStamped,

'/cmd_vel',

self.cmd_vel_callback,

10)

try:

self.arduino = serial.Serial('/dev/ttyUSB1', 9600, timeout=1)

time.sleep(2)

self.get_logger().info('✅ Connected to Arduino on /dev/ttyUSB1')

except Exception as e:

self.get_logger().error(f'❌ Failed to connect to Arduino: {e}')

def cmd_vel_callback(self, msg):

twist = msg.twist

linear = twist.linear.x

angular = twist.angular.z

command = f"{linear:.2f},{angular:.2f}\n"

self.arduino.write(command.encode())

self.get_logger().info(f"Sent: {command.strip()}")

def main(args=None):

rclpy.init(args=args)

node = ArduinoBridge()

rclpy.spin(node)

node.destroy_node()

rclpy.shutdown()

if __name__ == '__main__':

main()